# User manual

Nano Board with STM32F303CCT6 MCU

### Introduction

The Nano board allows users to easily develop applications with the STM32F303CCT6 microcontroller with the Arm Cortex-M4 32-bit core.

Based on STM32F303CCT6, it includes one MPU6050 (is a 3-axis gyroscope and a 3-axis accelerometer sensor), LEDs, push-buttons and a mini USB port.

Figure 1: Nano

Contents

- Features		                                                  				7

- List of tables								4

- List of figures								5

- Hardware and layout							8

List of tables

Table 1. 	Boot modes						10

Table 2.	LSE pins						11

Table 3.	HSE pins						11

Table 4.	UART1 pins						11

List of figures

Figure 1. 	Nano							1

Figure 2.	Nano top layout					8

Figure 3. 	Nano bottom layout					9

List of Block-diagram

Block 1:	UART-USB bridge					12

List of abbreviation

SWD		Serial Wire Debug

LQFP		Low Profile Quad Flat Package

LSE 		Low Speed External

HSE		High Speed External

Feature:

The Nano offers the following features:

- The STM32F303CCT6 microcontroller features a 32-bit Arm Cortex-M4 with FPU core (72 MHz max), 256-Kbyte Flash memory, and 48-Kbyte SRAM in a LQFP48 package.

- Three push buttons:

  - SW1(reset), SW2(user button), SW3(boot0).

- Four LEDs:

  - D1 (red) for 3.3 V power on / off.

  - Three user LEDs, D2 (blue), D3 (green), D4 (red).

- Board connectors:

  - J1 SWD.

  - J2 USB-B Micro.

  - 2.54 mm pitch extension header for 30 pins of LQFP48 I/Os for quick connection to prototyping board and easy probing.

- Power-supply options: ST-LINK or USB VBUS.

Hardware and layout

The Nano is designed around the STM32F303CCT6 microcontroller in a 48-pin LQFP package.

Figure 2 and Figure 3 help users to locate STM32F303CCT6 and its peripherals (MPU6050, push buttons, LEDs).

Figure 2: Nano top layout

Figure 3: Nano bottom layout

Power supply and power selection

- The power supply is provided either by the host PC through the USB cable or via the SWD port using an ST-Link connection.

  - U2 regulates the 5V input from USB to provide a steady 3.3V output.

Note: Don’t connect external power supply to header sockets.

LEDs

- D1 PWR: red LED indicates that the board is powered.

- User D2: blue LED is a user LED connected to the I/O PA1 of the STM32F303CCT6.

- User D3: green LED is a user LED connected to the I/O PA2 of the STM32F303CCT6.

- User D4: red LED is a user LED connected to the I/O PA3 of the STM32F303CCT6.

Push buttons

- SW1 RESET: Push button connected to NRST is used to RESET the STM32F303CCT6.

- SW2 USER: User button is connected to the I/O PA0 of the STM32F303CCT6.

- SW3 BOOT0: Push button connected to BOOT0 is used to toggle the boot mode of the STM32F303CCT6.

| BOOT0 | Boot Mode |
| --- | --- |
| 0 | Main flash memory |
| 1 | System memory |

Table 1: Boot modes

Note: By default, the microcontroller runs the application code from the main flash memory. To change this behavior, use the BOOT0 button.

- Press the BOOT0 button and then press the reset button.

- This action makes the microcontroller to run the built-in bootloader from the system memory (i.e., microcontroller enters the bootloader mode).

6-Axis (Gyro + Accelerometer) MEMS Motion Tracking

- MPU6050 sensor is a low power, low cost, and high-performance 6-axis (Gyro + Accelerometer).

- The MPU6050 devices combine a 3-axis gyroscope and a 3-axis accelerometer on the same silicon die, together with an onboard Digital Motion Processor (DMP), which processes complex 6-axis Motion Fusion algorithms.

- The STM32F303CCT6 microcontroller controls this sensor through the I2C interface.

OSC Clock

- LSE: OSC 32.768 kHz clock supply

Refers to an external oscillator running at 32.768 kHz. It typically provides a low-frequency clock source for real-time clocks (RTC) or other timing-sensitive functions.

| Pin Name | Pin Function |
| --- | --- |
| PC14 | OSC32_IN |
| PC15 | OSC32_OUT |

Table 1: LSE pins

- HSE: OSC 8 MHz clock supply

Refers to an external oscillator running at 8MHz. It provides a higher-frequency clock source suitable for driving the core processing unit or other high-speed peripherals.

| Pin Name | Pin Function |
| --- | --- |
| PF0 | OSC_IN |
| PF1 | OSC_OUT |

Table 2: HSE pins

USB to UART bridge

- The USB to UART bridge facilitates communication between a computer and a STM32F303CCT6, with UART1Tx and UART1Rx serving as the transmit and receive pins, respectively, connected to pins PA9 and PA10 on the microcontroller.

| Pin Name | Pin Function |
| --- | --- |
| PA9 | UART1_Tx |
| PA10 | UART1_Rx |

Table 3: UART1 pins

TO PC

CH340N-U5

STM32F303CCT6 - U1

TX

USART1_RX PA10

USART1_TX PA9

RX

Block1: UART-ISB Bridge